CS 350 Final Project: Smart Thermostat Architecture Report

Kylie Reiter | August 16, 2026

SysTec's smart thermostat needs an architecture that supports GPIO, I2C, UART, PWM, and an LCD, connects to the cloud through Wi-Fi, and has enough Flash and RAM for the application. These requirements guide the comparison of Raspberry Pi, Microchip, and Freescale/NXP architectures.

The Raspberry Pi Pico W provides 26 multifunction GPIO pins, two UARTs, two I2C controllers, two SPI controllers, and 16 PWM channels. It also has 264 KB SRAM, 2 MB onboard flash, and integrated 2.4 GHz 802.11b/g/n Wi-Fi (Raspberry Pi Ltd., 2025). These resources support the thermostat's sensor, buttons, LEDs, and display. Wi-Fi allows the production thermostat to send temperature, set-point, and state data directly to SysTec's cloud server instead of using the prototype's UART connection.

Microchip's WFI32E01 includes a 200 MHz MCU, 1 MB embedded flash, 256 KB program/data SRAM plus a 64 KB data buffer, 37 GPIOs, three UARTs, I2C, and two SPI interfaces (Microchip Technology Inc., 2026). These peripherals support the thermostat hardware, while integrated 2.4 GHz 802.11b/g/n Wi-Fi with WPA3 and TLS can securely transmit thermostat data to cloud services. Its Flash and RAM support the application, network stack, and cloud communication.

Freescale is now represented by NXP. NXP's RW612 uses a 260 MHz Arm Cortex-M33 processor, provides 1.2 MB SRAM, supports external flash through Quad FlexSPI, and includes UART, SPI, I2C, GPIO, timers/PWM, and an LCD interface (NXP Semiconductors, 2024). NXP lists thermostats as a smart-home application. Integrated dual-band Wi-Fi 6 can connect directly to SysTec's cloud services without a separate Wi-Fi controller, while its memory and peripheral capacity leave room for future features.

All three architectures meet SysTec's core requirements. The Pico W is accessible and capable, and the WFI32E01 offers strong embedded networking and security. I recommend the NXP RW612 for production because its Wi-Fi 6, 1.2 MB SRAM, external flash support, required peripheral interfaces, and smart-home focus provide the best combination of connectivity and expansion capability for a commercial cloud-connected thermostat.

References

Microchip Technology Inc. (2026). WFI32E01PC. https://www.microchip.com/en-us/product/wfi32e01pc

NXP Semiconductors. (2024). RW612 wireless MCU [Data sheet]. https://www.nxp.com/docs/en/data-sheet/RW612.pdf

Raspberry Pi Ltd. (2025). Raspberry Pi Pico W product brief. https://www.raspberrypi.com/products/raspberry-pi-pico/